| CHARACTER： 1.Physical performance a、Excellent mechanical properties b、Excellent thermal stability c、Low temperature soft good performance d、Good flame retardancy e、Of high resistance to chemicals f、Chemistry stability is good b、Good resistance to high temperature of the engine fluid j、Excellent resistance to impact ability 2.Electrical Properties a、Electrical insulation good b、Good the dielectric properties 3.Processing properties a、Using the hot extrusion processing b、Can be twisted pair and multi-core c、Good processing properties Harness d、Harness processing process good compatibility e、According to UL standard design 4.Environmental protection a、ROHS/REACH compliant SHOULD BE USED： Internal or external wiring in Class 2 circuits of computers and electronic equipment. REFERENCE： UL758-2010、UL1581-2009 Outline： . | Strands Tinned/Bare Copper Shield PVC Insulation PVC Jacket Internal or external wiring in Class 2 circuits of computers and electronic equipment. UL2448. Wire structure description： Conductor: Tinned /Bare copper ； Insulation materials: PVC Insulation Shield materials:tinned/bare copper or Aluminum foil warp (optional) Jacket materials: PVC jacket. Internal or external wiring of electronic equipment ，More than 60 ℃ environment temperature occasion Rated temperature: 60℃ Rated voltage:30V. Part No.; Conductor; Insulation thickness (mm); Jacket Thickness (mm); Overall diameter (mm); Conductor resistance 20℃ （Ω/Km）. Cores No.; AWG; N0./ mm; Dia. mm; Nom.; Min Dia.; Nom.; Min; Nom.; min. 2448; 2; 26; 7/0.16; 0.48; 0.25; 0.98; 0.76; 0.61; 3.88; 3.58; 150. 2; 24; 11/0.16; 0.61; 0.25; 1.11; 0.76; 0.61; 4.14; 3.84; 94.2. 2; 22; 17/0.16; 0.76; 0.25; 1.26; 0.76; 0.61; 4.43; 4.13; 59.4. 2; 20; 26/0.16; 0.95; 0.25; 1.45; 0.76; 0.61; 4.82; 4.52; 36.7. 3; 26; 7/0.16; 0.48; 0.25; 0.98; 0.76; 0.61; 3.88; 3.58; 150. 3; 24; 11/0.16; 0.61; 0.25; 1.11; 0.76; 0.61; 4.14; 3.84; 94.2. 3; 22; 17/0.16; 0.76; 0.25; 1.26; 0.76; 0.61; 4.43; 4.13; 59.4. 3; 20; 26/0.16; 0.95; 0.25; 1.45; 0.76; 0.61; 4.82; 4.52; 36.7. 4; 28; 7/0.127; 0.38; 0.25; 0.88; 0.76; 0.61; 3.95; 3.65; 239. 4; 26; 7/0.16; 0.48; 0.25; 0.98; 0.76; 0.61; 4.18; 3.88; 150. 5; 28; 7/0.127; 0.38; 0.25; 0.88; 0.76; 0.61; 4.19; 3.89; 239. 5; 26; 7/0.16; 0.48; 0.25; 0.98; 0.76; 0.61; 4.45; 4.15; 150. 6; 28; 7/0.127; 0.38; 0.25; 0.88; 0.76; 0.61; 4.41; 4.11; 239. 6; 26; 7/0.16; 0.48; 0.25; 0.98; 0.76; 0.61; 4.69; 4.39; 150. 7; 28; 7/0.127; 0.38; 0.25; 0.88; 0.76; 0.61; 4.61; 4.31; 239. 7; 26; 7/0.16; 0.48; 0.25; 0.98; 0.76; 0.61; 4.91; 4.61; 150. Note:shield thickness 0.20mm. Marking：E211048 AWM STYLE 2448 NO. AWG 60℃ 30V VW-1 PVC QIFURUI c AWM I A 60℃ 30V FT1 –LF- SAE COLOR SERIES * STOCK COLOR CHART. 00-BLACK; 01-WHITE; 02-RED; 03-YELLOW; 04-GREEN. 05-BLUE; 06-BROWN; 07-GREY; 08-ORANGE; 09- VIOLET. PACKAGE *PACKAGE. Part No.; Packing- Ft/roll. 2～7; □ 100Ft; □ 500Ft; ■ 1000Ft. According to customer requirements for packaging packaging. |
| --- | --- |